Model Question Paper-2006

NATIONAL CERTIFICATION EXAMINATION

FOR

ENERGY MANAGERS & ENERGY AUDITORS

PAPER – 2: Energy Efficiency in Thermal Utilities Duration: 3 HRS Max. Marks: 150

# Section – I:	 OBJECTIVE TYPE 				Marks: 50 x 1 = 50

(i)	Answer all 50 questions

(ii)	Each question carries one mark

(iii)	Put a () tick mark in the appropriate box in the answer book

| 1. | Which of the following fuel needs preheating for pumping from storage tank a) LSHS b) Furnace Oil c) Heavy Fuel Stock d) All of the above |
| --- | --- |
| 2. | Pour point of the fuel is the ……. a) Lowest temperature at which it will flow when cooled from high temperature b) High temperature at which it will flow when heated from low temperature c) Lowest temperature at which it will solidify when cooled from high temperature d) Highest temperature at which it will solidify when cooled from high temperature |
| 3. | Sulfur content is lowest in a) LSHS b)LDO c) Furnace oil d) Kerosene |
| 4. | Calorific value of fuel is highest in - Furnace oil b) LSHS c) LDO d) Anthracite coal |
| 5. | Spontaneous combustion can be prevented by a) Allowing more ventilation of air b) Spraying water over the leaps of coal c) Both a and b d None of the above |
| 6. | Higher the sulphor content in the fuel, Dew point temperature will a) decrease b) increase c) same d) No impact |
| 7. | Which of the following is recommended for pumping light fuel oil a) Diaphragm Pump b) Gear Pump c) Centrifugal Pump d) All of the above |
| 8. | High ash content will a) Increase the handling cost b) Causes clinkering and Slagging c) Reduces boiler capacity d) All of the above |
| 9. | Blending of coal means a) Mixing of excessive fines with large lumps of coal b) High GCV coal with Low GCV coal c) Both a and b d) Mixing high sulphor content with low sulphor content |
| 10. | Moisture content is less in a) Deoiled Bran b) Paddy Husk c) Saw Dust d)Coconut Shell |
| 11. | The atomization temperature of 105oC is recommended for - LAP Burner c) HAP Burner - MAP Burner d) All of the above |
| 12. | The ash fusion temperature for typical Indian coal is a) 1700oC b) 1300oC c) 1100oC d) 900oC |
| 13. | The temperature of flue gas is high in a) Two pass Boiler b) Three Pass Boiler c) Both a and b d) None of the above |
| 14. | Steam pipes comes under IBR Inspection if a) Steam pressure exceeds 5.5kg/m2 and internal diameter exceeds 250mm b) Steam pressure exceeds 3.5kg/m2 and internal diameter exceeds 254mm c) Steam pressure exceeds 1.5 5kg/m2 and internal diameter exceeds25mm d) None of the above |
| 15. | Pulverized coal is crushed into a) 50 to 55% is below 75 microns b) 60-65% is below 75 microns c) 70-75% is below 75 microns d) 90-95% is below 75 microns |
| 16 | Suspension burning as well as Grate burning takes place in a) Chain Grate Stoker boiler b) Spreader Stoker boiler c) Pulverized fuel boiler d) Fluidized Bed boiler |
| 17 | For complete combustion 1kg sulfur needs theoretically a) 5.45kg of air b) 4.35kg of air c) 3.45kg of air d) None of the above |
| 18 | Highest permissible TDS limit is allowed for a) Economic boiler b) Water tube boiler c) Smoke tube boiler d) Lancashire boiler |
| 19 | What is the % of Excess air if CO2% measured is 7.5 and theoretical CO2 is 15% a) 50% b) 75% c) 100% d) None of the above |
| 20. | The TDS level by Softening process method for boiler water treatment may be a) reduced b) increased c) Both a and b d) None of the above |
| 21 | Semi permeable membrane will allow to a) Flow of minerals and molecules only b) Flow of water molecules only c) Both a and b d) None of the above |
| 22 | The operating Excess air percentage is high in a) Spreader Stoker coal fired boiler b) Oil fired boiler c) Bagasse fired boiler d) Wood fired boiler |
| 23 | Boiler replacement may be recommended if boiler is a) Old and inefficient b) Over sized for present conditions c) Not designed for ideal loading conditions d) All of the above |
| 24 | Specific volume of steam increases a) With increasing pressure b) With decreasing pressure c) Does not changes with pressure d) None of the above |
| 25 | Latent heat at 1 atm pressure is a) Zero b) 540 kcal/kg c) Infinite d) None of the above |
| 26 | From the steam header, condensate is to be drained for every a) 10 meter length steam pipe line b) 20 meter length steam pipe line c) 30 meter length steam pipe line d) 50 meter length steam pipe line |
| 27 | It is preferable to choose the type of…….. trap for draining condensate from steam header pipeline is a) TD Trap b) Inverted Bucket Trap c) a or b d) Float trap |
| 28 | Saturated steam will flow at velocity of a) Upto 40 km/hr b) Upto 70 km/hr c) Upto 100 km/hr d) Upto 140 km/hr |
| 29 | Flash steam can be identified by following a) Steam blows out continuously in a blue stream b) Steam blows out continuously in a white stream c) Steam blows out in Blue d) Steam blows out in white intermittently |
| 30 | Self Recuperative burner used to a) Preheat the fuel oil b) Preheat the combustion air c) Both a and b d) None of the above |
| 31 | A Refractory property, PCE is called a) Preheated cone equivalent b) Pyrometric cone equivalent c) Pressurized Cone equilibrium d) None of the above |
| 32 | Higher the bulk density of a given refractory, increases a) its volume stability b) its heat capacity c) its resistance to slag penetration d) All of the above |
| 33 | Cold crushing strength is high for a) Clay refractory b) High Alumina refractory c) Diatomite solid grade d) Silica refractory |
| 34 | Out of 3 Ts of combustion, the fluidized bed combustion mostly utilizes………and ………….for good combustion a) Time and Temperature b) Temperature and turbulence c) Time and turbulence d) None of the above |
| 35 | The size of crushed coal for CFBC boiler a) 1-10 mm b) 3-12 mm c) 6-10 mm d) 5-12 mm |
| 36 | The overall conversion efficiency of PFBC boiler is higher by ----- % over conventional power plant a) ) 2-5% b) 3-6% c) 4-7% d) 5-8% |
| 37 | Which of the following is not dependency factor for in bed heat transfer of FBC Boiler a) Particle size b) Bed temperature c) Bed pressure d) Primary air velocity |
| 38 | The amount of fly ash to be handled in FBC boiler is a)Low when compared to conventional boiler b)Medium when compared to conventional boiler c)High when compared to conventional boiler d)Same when compared to conventional boiler |
| 39 | Which of the Cogen system can offer a minimum range of heat to power ratios a) Back pressure steam turbine b) Combined Cycle c) Gas turbine d) DG sets |
| 40 | Cogeneration system is most suitable for a) Sugar Industry b) Foundry c) Automobile Engineering d) Oil extraction Industry |
| 41 | Best suited fuel for Gas turbine a) HSD b) Naphtha c) LSHS d) Kerosene |
| 42 | Which of the following gives max electrical efficiency a) Large gas turbine b) Reciprocating engines c) Condensing steam turbine d) Back pressure steam turbine |
| 43 | A 2000 kg/hr flue gas is exhausted from DG set at 400oC. Specific heat is 0.25 kcal/kg. Ambient temperature is 30oC. What is the Heat available in the flue gas? a) 2,00,000 kcal/hr b) 1,85,000 kcal/hr c) 8,00,000 kcal/hr d) 1,00,000 kcal/hr |
| 44 | Heat recovery from the Dryer exhaust air is a good application of……….in industry a) Heat Pump b) Heat Wheels c) Economizer d) Refrigerator |
| 45 | Thermo compressor is used for a) Recovery of flue gas at low temperature b) Recovery Flash steam from condensate c) Recovery Vapour from hazards chemical process d) All of the above |
| 46 | The maximum efficiency possible for sensible heat recovery in heat wheel is a) 25% b) 50% c) 85% d) 95% |
| 47 | HP HE means a) Heat Pump Heat Exchanger b) Heat Pipe Heat Exchanger c) High Pressure Heat Exchanger d) None of the above |
| 48 | Ceramic recuperators can be used when the exhaust flue gas temperature available a) Upto 600 b) 600 to 1000 c) More than 1300oC d) None of the above |
| 49 | Maximum Heat transfer occurred in heat treatment furnace process is by a) Radiation b) Convection c) Conduction d) Convection & Conduction |
| 50 | 20% make up water is used In an Economic Boiler. The TDS of make up water and blow down water are 200 ppm and 3000 ppm respectively. What is the blow down rate? a) 1.00 b) 1.25 c) 2.00 d) 2.25 |

# Section - II: 	  SHORT DESCRIPTIVE QUESTIONS		Marks: 10 x 5 = 50

(i)	Answer all Ten questions

(ii)	Each question carries Five marks

S-1	A 60 kg/hr of fuel oil (LDO) was fired in process boiler.  The actual and 	theoretical 	percentage of carbon-dioxide is 9 and 16% respectively. Assume Theoretical 	air 	required for fuel oil is 14.The specific gravity of oil is 0.88.

Find out the Excess air and calculate the quantity of flue gas

S-2	List down ten energy conservation measures in steam system.

S-3	Explain the features of packaged boilers and its features.

S-4	In agriculture based industry; wood with 30% moisture was burned in a boiler.  The GCV of wood was 2400 kcal/kg.  Energy Manager has suggested to dry the wood in open Sun drying to reduce the moisture content from 30% to 10% for wood saving.  Flue gas temperature is 180oC.  Ambient temperature is 30oC.  How much wood can be saved by this suggestion?

S-5	Discuss Group trapping and Individual trapping.

S-6	Classify the various types of insulation as per temperature and give an example for each type.

S-7	Briefly explain mechanical type De-aeration.

S-8	Flash steam is generated at 2 bar from 7 bar condensate water.

Enthalpy of 7 bar condensate                     = 165.6 kcal/kg

Enthalpy of 2 bar condensate                     = 120 kcal/kg

Total heat content in flash and steam         = 646 kcal/kg

Find out the % of flash steam generation.

S-9	A batch type heat treatment furnace was operating 8 hrs. per day in a foundry industry.  Determine the Heat Loss due to furnace opening which is used for charging and discharging from the given data

Area of door opening			= 1.5 m2

Factor of total radiation		= 0.65

Furnace operating Temperature        = 1200oC

Ambient temperature                         = 37oC

S-10	What are the Benefits of WHR devices implementation in Industry?  List down five 	WHR sources in a typical industry.

-------- End of Section  - II ---------

# Section - III: 	  LONG DESCRIPTIVE QUESTIONS                Marks: 5 x 10 = 50

(i)	Answer all Five questions 	(ii)	Each question carries Ten marks

L-1	 What is the predominant Hydrocarbon present in the LPG?  Calculate the Theoretical air required for complete combustion of 1 kg of propane with 0.04% Oxygen.

L-2	Determine the boiler efficiency by direct method and evaporation ratio from the 	following data.

Boiler capacity Rating				=  10 TPH, F & A 100oC

Fuel oil (LSHS) consumption           	            =   600 lit/hr

Specific gravity of fuel oil                                     =   0.90

Saturated steam generation pressure &

Temperature                   =   8 kg/cm2(a) and 170oC

Qty. of steam generation                                      =   8 TPH with 0.9 dryness fraction

Enthalpy of steam			           =   660.8 kcal/kg at 8 kg/cm2(a)

Feed water temperature			           =   60oC

GCV of fuel oil				           =   10600 kcal/kg

L-3	Find out the pressure drop occurred in 500 meters length of steam pipelines.  The bore of the pipe is 250 mm in horizontal piping.  Saturated steam velocity is 20 m/s and friction factor is 0.005.  How much pressure drop will be reduced if pipe dia has increased from 250 mm to 350 mm.

L-4	Briefly explain the factors influencing cogeneration choice.  For a particular site specific industry.

L-5	What are the criteria for development of waste heat recovery system in an industry.  Explain the working principle and application of Heat pipes.

End of Section  - III ---------